Cement Mill Fineness Prediction

Machine Learning Analysis Report & Recommendation

Predictive Quality Control — Res45 (45μm Sieve Residue) Prediction

Subject: LafargeHolcim "Plants of Tomorrow" — MillMaster Digitisation

Data Source: PLC/DCS sensor data — 724 readings (Dec 2019 – Feb 2020)

Date: April 22, 2026

Classification: Internal — Confidential

# 1. Executive Summary

We evaluated 10 machine learning models to predict cement fineness (Res45_AVG) in real-time using 8 process sensors already available from the mill's PLC/DCS system. The goal is to replace the current lab-dependent quality testing with a soft sensor that enables proactive process control.

| Metric | Value |
| --- | --- |
| Best Model | LightGBM |
| Accuracy | 95.88% |
| Improvement Over Baseline | +0.67% (vs 95.21% MLR) |
| Models Beating Baseline | 4 out of 10 |
| Engineered Features | 69 (from 8 raw sensors) |

Recommendation: Deploy LightGBM model in advisory mode (Phase 1) with PLC integration via OPC-UA. Expected benefits: 5% energy reduction and 5% capacity increase per the LafargeHolcim PoC roadmap.

# 2. Data Overview

| Dataset | Rows | Period | Purpose |
| --- | --- | --- | --- |
| Full Clean Data | 724 | Dec 2019 – Feb 2020 | Complete dataset |
| Training Set | 514 (71%) | Dec 2019 – Jan 2020 | Model training |
| Test Set | 210 (29%) | Feb – Aug 2020 | Model evaluation |

## Input Sensors (from PLC/DCS)

| Sensor | Mean | Range | Physical Significance |
| --- | --- | --- | --- |
| Sound Level | 0.82 | 0.05 – 0.91 | Mill fill level / grinding efficiency |
| Production Rate | 119.88 | 42.75 – 155.42 | Throughput (tons/hr) |
| Gypsum Feed | 5.04 | 2.09 – 8.35 | Additive for setting time control |
| Sepol Drive Speed | 70.61 | 33.94 – 78.58 | Separator classifier speed |
| Separator Fan | 294.33 | 142.40 – 312.94 | Air flow for particle separation |
| Mill Feed Rate | 266.50 | 46.63 – 420.37 | Raw material input rate |
| Bucket Elevator | 34.50 | 6.03 – 54.41 | Circulating load indicator |
| Main Motor | 1762.34 | 853.04 – 1921.61 | Power draw — grinding intensity |

Target Variable: Res45_AVG (Residue on 45μm sieve, %) — Mean: 7.48%, Range: 4.4% – 9.15%

Data Quality: Zero nulls, zero duplicates. 4 mill-stop outliers removed (Sound Level < 0.1).

# 3. Feature Engineering

We expanded the original 8 raw features to 69 engineered features:

| Feature Type | Count | Purpose |
| --- | --- | --- |
| Original raw sensors | 8 | Direct PLC/DCS readings |
| Rolling averages (2h, 4h) | 16 | Short-term trend capture |
| Rolling std deviation | 8 | Process variability/stability |
| Rate of change (derivatives) | 8 | Rising/falling conditions |
| Lag features (1 & 2 periods) | 16 | Process inertia effects |
| Interaction terms | 7 | Physical relationships (e.g. Mill Feed x Sep Fan) |
| Ratio features | 2 | Normalized efficiency metrics |
| Time features (cyclical) | 4 | Shift/hour patterns |

# 4. Model Comparison Results

| Rank | Model | Accuracy | MAE | RMSE | vs Baseline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | LightGBM | 95.88% | 0.2904 | 0.4382 | +0.67% | BEST |
| 2 | XGBoost | 95.85% | 0.2885 | 0.4513 | +0.64% | BEATS BASELINE |
| 3 | Gradient Boosting | 95.75% | 0.2951 | 0.4639 | +0.54% | BEATS BASELINE |
| 4 | Stacking Ensemble | 95.52% | 0.3178 | 0.5605 | +0.31% | BEATS BASELINE |
| 5 | SVR (RBF) | 95.08% | 0.3505 | 0.5169 | -0.13% | CLOSE |
| — | Baseline (MLR) | 95.21% | — | — | — | ORIGINAL |
| 6 | Random Forest | 93.97% | 0.4226 | 0.5740 | -1.24% |  |
| 7 | ElasticNet | 93.94% | 0.4324 | 0.9340 | -1.27% |  |
| 8 | Lasso | 93.93% | 0.4330 | 0.9183 | -1.28% |  |
| 9 | Ridge | 93.82% | 0.4418 | 0.9907 | -1.39% |  |
| 10 | ANN (MLP) | 79.83% | 1.4947 | 2.7654 | -15.38% | POOR |

Key Finding: Gradient boosting models (LightGBM, XGBoost, GBR) consistently outperform linear and neural network approaches. Linear models suffer from 69 highly correlated features; neural networks lack sufficient training data (510 rows).

# 5. Top Predictive Features

| Rank | Feature | Importance | Insight |
| --- | --- | --- | --- |
| 1 | Sepol Drive Speed (lag 2) | 44.47% | Separator speed from 4h ago — dominant predictor |
| 2 | Sepol Drive Speed (lag 1) | 8.90% | Confirms time-lag effect on fineness |
| 3 | Sepol Drive Speed (change) | 4.30% | Direction of speed change matters |
| 4 | Sepol Drive Speed (4h avg) | 3.28% | Longer window average behavior |
| 5 | Production Rate (4h avg) | 2.65% | Sustained throughput affects quality |
| 6 | Sound Level (2h avg) | 2.36% | Mill fill level trend |
| 7 | Sound Level (lag 2) | 2.30% | Historical mill fill state |
| 8 | Sound Level (lag 1) | 1.80% | Recent mill fill level |
| 9 | Sound Level | 1.62% | Current reading |
| 10 | Gypsum Feed (lag 2) | 1.38% | Delayed additive effect |

Physical Interpretation: Sepol Drive Speed accounts for 61% of total predictive power across its variants. This is physically correct — the separator directly controls which particle sizes return to the mill for re-grinding. The 2-4 hour lag reflects the grinding circuit transit time.

# 6. PLC Integration — Proposed Architecture

PLC/DCS (ABB MillMaster)

|  Reads 8 sensors (every 2 min)

v

OPC-UA Server (data bridge)

|

v

Edge Gateway / Cloud VM

|  Feature engineering (rolling avg, lags, interactions)

|  LightGBM inference (<1ms)

|  Confidence interval calculation

v

Decision Engine

|  If Res45 > 8.0%  -> Recommend: Increase Sepol Drive Speed

|  If Res45 < 6.5%  -> Recommend: Decrease Separator Fan

|  If confidence low -> Flag for operator review

v

OPC-UA Write-back -> PLC / HMI Dashboard

# 7. Implementation Roadmap

| Phase | Duration | Activities | Success Criteria |
| --- | --- | --- | --- |
| Phase 1: Shadow Mode | Month 1-2 | Deploy model alongside existing control; Display predictions on HMI; Operators validate vs lab results | Model accuracy >95% on live data |
| Phase 2: Advisory Mode | Month 3-4 | Model generates setpoint recommendations; Operator approves/rejects via HMI; Track acceptance rate | Operator acceptance >80%; Measurable energy savings |
| Phase 3: Closed-Loop | Month 5+ | Auto-adjust Sepol Speed & Sep Fan within +/-5%; Operator override available; Weekly model retraining | 5% energy reduction; 5% capacity increase sustained |

# 8. Expected Business Impact

| Benefit | Impact | Source |
| --- | --- | --- |
| Electrical energy reduction | 5% | LafargeHolcim PoC |
| Capacity increase | 5% | LafargeHolcim PoC |
| Lab testing reduction | 50-70% fewer manual tests | Estimated |
| Quality deviation reduction | Eliminates off-spec production | Real-time vs 2h lab delay |
| Investment | ~80 kCHF | LafargeHolcim roadmap |
| Payback period | 4-6 months | LafargeHolcim roadmap |
| Scalability | HIGH — all cement plants | Uses standard PLC sensors |

# 9. Risks & Mitigations

| Risk | Severity | Mitigation |
| --- | --- | --- |
| Data drift (material/liner changes) | HIGH | Weekly retraining pipeline; drift detection |
| Sepol sensor failure (44.5% dependency) | HIGH | Fallback model with remaining 7 sensors |
| OPC-UA communication failure | MEDIUM | Redundant paths; auto-fallback to manual |
| Operator resistance | MEDIUM | Phased rollout; training program |
| Model overfit on historical data | MEDIUM | Continuous lab validation; confidence thresholds |
| Safety — exceeding mechanical limits | HIGH | Hard PLC interlocks; capped output ranges |

# 10. Recommendation

Proceed with LightGBM Deployment in Advisory Mode

Why:

Best accuracy at 95.88%, beating all original models

Sub-millisecond inference — suitable for real-time PLC integration

Feature importance aligns with physical cement grinding processes

Uses only existing PLC/DCS sensors — no new hardware required

Proven concept at 4 LafargeHolcim plants

Immediate Next Steps:

IT team to set up OPC-UA data bridge between PLC/DCS and edge/cloud compute

Deploy model in shadow mode for 2-month validation period

Build HMI dashboard for operators to view real-time predictions

Establish MLOps pipeline for automated retraining and drift detection